NATIONAL INSTITUTE OF BUSINESS MANAGEMENT

Higher National Diploma in Software Engineering

PROJECT NYX

An Interactive Arduino-Based Robotic Pet

Final Project Report

Submitted by

A. G. Oshan Tanusha

Batch 26.1F

2026

# Abstract

Project NYX is an interactive robotic pet that I built around an Arduino Nano to explore how a small embedded system can sense its surroundings and respond to them in a way that feels alive rather than purely mechanical. The finished prototype pairs an HC-SR04 ultrasonic sensor with two servo motors for pan-and-tilt head movement and a 128 x 64 SSD1306 OLED screen that acts as the robot's face. When something moves within range, NYX notices it, changes its expression and turns its head toward it, which is the core behaviour the whole project was built to demonstrate.

The system was not built in one pass. It grew through several versions: V0 proved out the basic idea of sensing distance and driving a servo, V1 added proper behavioural states and smoother movement, and V2 introduced more facial expressions along with infrared edge detection. V3 was meant to bring everything together with full body movement on wheels, but the motor driver and part of the movement power system were damaged partway through development, and the original ultrasonic and IR modules were also unavailable for a stretch of the build. Because of that, the version demonstrated for this report is a hardware-limited V3: the ultrasonic sensing, OLED expressions and head movement are all real and working, while the wheel movement is disabled until the damaged hardware can be replaced.

Even with that limitation, the project met its main goal, which was to show that sensing, actuation, a display and embedded software can be combined into one coherent, interactive robotic platform rather than a collection of separate demos.

# Acknowledgement

I would like to thank the lecturers and staff of the National Institute of Business Management for their guidance throughout the HND in Software Engineering programme, and in particular for the support and feedback given during the development of Project NYX. I am also grateful to my batch mates in 24.3F for their encouragement and for the many practical suggestions that helped me troubleshoot problems along the way.

Finally, I would like to thank my family for their patience and support, especially during the periods when hardware issues set the project back and the late nights that followed to get it back on track.

# Table of Contents

Abstract	1

Acknowledgement	1

Table of Contents	1

Chapter 01 – Introduction	1

1.1 Introduction	1

1.2 Background of the Project	1

1.3 Problem Statement	1

1.4 Aim	1

1.5 Objectives	1

1.6 Scope	1

Chapter 02 – Literature Review	1

2.1 Robotics	1

2.2 Embedded Systems	1

2.3 Ultrasonic Sensors	1

2.4 Servo Motors	1

2.5 OLED Displays	1

Chapter 03 – System Design	1

3.1 System Architecture	1

Input	1

Processing	1

Output	1

3.2 Hardware Configuration	1

3.3 Software Architecture	1

Chapter 04 – Implementation	1

4.1 Arduino Programming	1

4.2 Ultrasonic Distance Measurement	1

4.3 Object Detection	1

4.4 Servo Movement	1

4.5 OLED Facial System	1

4.6 Body Movement	1

Chapter 05 – Testing and Results	1

5.1 Testing Methodology	1

5.2 Ultrasonic Testing	1

5.3 Servo Testing	1

5.4 OLED Testing	1

5.5 Integrated Testing	1

5.6 Testing Limitations	1

Chapter 06 – Conclusion and Future Development	1

6.1 Conclusion	1

6.2 Achievements	1

6.3 Limitations	1

6.4 Future Improvements	1

6.5 Final Conclusion	1

References	1

Appendix A – Current V3 Demo Pin Configuration	1

Appendix B – V3 Operating Logic	1

# Chapter 01 – Introduction

## 1.1 Introduction

Robotics sits at the intersection of electronics, mechanical design, programming and control theory, and one of the more interesting shifts in the field over the past decade has been the move away from robots that simply carry out fixed mechanical tasks toward robots that are expected to notice and react to what is happening around them.

Project NYX is my attempt to build a small-scale example of that idea. It is an interactive robotic prototype, controlled by an Arduino microcontroller, that is meant to demonstrate three things working together: sensing the environment, moving in response to it, and communicating that response visually. Rather than building a robot that only measures distance and prints a number, I wanted NYX to feel like it was reacting – turning its head, changing its expression – the way a curious pet might.

## 1.2 Background of the Project

Most beginner and intermediate robotics projects use a sensor to read the environment and an actuator to act on that reading, which is a good starting point but tends to leave out any feedback about what the robot is actually “thinking” or doing. I felt that pairing sensing with expressive feedback, in this case a small face on an OLED screen, makes a robot noticeably easier to understand and more engaging to interact with, even though the underlying logic is still fairly simple.

I developed NYX in stages rather than trying to build the full design at once. The earliest version focused only on getting distance detection, servo movement and OLED output working together. Later versions built on that foundation with more expressions, smoother head motion and more considered behaviour logic.

The original plan for V3 combined all of that with a wheeled body so NYX could move around and not just look around. Unfortunately, the power system driving that movement was damaged during development, so the version I am presenting focuses on real ultrasonic sensing and head movement, with body movement disabled rather than removed.

## 1.3 Problem Statement

A lot of introductory robotics prototypes can detect an object or turn a motor, but they rarely give any indication of what the robot has detected or how it is responding – the interaction usually stops at a number on a serial monitor or a motor turning without explanation.

Project NYX addresses this by combining environmental sensing with physical head movement and OLED facial expressions, so the robot's response to a nearby object is something a person can actually read and understand at a glance, instead of having to infer it from raw sensor output.

## 1.4 Aim

The aim of Project NYX is to design and build an interactive robotic system that can sense nearby objects, control its own movement, and communicate its current state through visual expression.

## 1.5 Objectives

- Develop an Arduino-based robotic control system.

- Detect nearby objects using an ultrasonic sensor.

- Control pan and tilt head movement using servo motors.

- Display robotic facial expressions using an OLED screen.

- Create responsive behaviour based on sensor input.

- Develop smooth, natural-looking servo movement.

- Integrate the hardware and software into one working robotic system.

- Develop a mobile robotic movement system as part of the complete V3 design.

- Develop a hardware-limited demonstration mode for situations where the body-movement hardware is unavailable.

- Test individual components before integrating them.

## 1.6 Scope

The project covers the following areas:

- Arduino programming

- HC-SR04 ultrasonic sensing

- Servo motor control

- OLED graphics

- Facial expressions

- Object detection

- Head movement

- Autonomous behaviour

- Robotic body movement design

- Component-level testing

- Hardware and software integration

Advanced artificial intelligence, computer vision, voice recognition and cloud connectivity are outside the current scope of this project.

# Chapter 02 – Literature Review

## 2.1 Robotics

At its core, robotics is about building machines that can sense their environment, process that information, and then act on it physically. What separates an “interactive” robot from a purely mechanical one is the addition of some communication channel – facial expression, sound, or deliberate movement – that lets a human observer make sense of the robot's response without needing to inspect its internal logic.

## 2.2 Embedded Systems

An embedded system is essentially a computer built to do one job well rather than run general-purpose software. In NYX, that job belongs to the Arduino microcontroller, which reads the ultrasonic sensor, decides what the robot should do, and drives the servos and OLED display accordingly.

## 2.3 Ultrasonic Sensors

The HC-SR04 ultrasonic sensor works by sending out a short ultrasonic pulse and timing how long it takes for the echo to bounce back. Distance is then calculated from that time using the formula below.

Distance = (Time × Speed of Sound) ÷ 2

The division by two is necessary because the measured time covers the round trip – the pulse travelling out to the object and the echo travelling back – not just the one-way distance.

## 2.4 Servo Motors

Servo motors give precise, controllable angular movement, and NYX uses two of them for its head:

- Pan servo – controls horizontal (left/right) head movement

- Tilt servo – controls vertical (up/down) head movement

Together, these let the robot “look” toward whatever it has detected.

## 2.5 OLED Displays

The SSD1306 OLED is a small, low-power graphical display, and in this project it doubles as NYX's face. Different pre-programmed graphics represent different states – normal, curious, happy, blinking and worried – giving the robot a simple but readable “personality”.

# Chapter 03 – System Design

## 3.1 System Architecture

NYX is organised around three functional sections: input, processing and output.

### Input

- HC-SR04 ultrasonic sensor

- IR sensors (part of the complete V3 design)

### Processing

- Arduino microcontroller

- Sensor data processing

- Behaviour decision logic

### Output

- Pan servo

- Tilt servo

- OLED display

- Motor system (part of the complete V3 design)

## 3.2 Hardware Configuration

The table below lists the pin configuration used in the current V3 demonstration build.

| Component | Arduino Pin |
| --- | --- |
| Ultrasonic Trigger | D2 |
| Ultrasonic Echo | D3 |
| Pan Servo | D9 |
| Tilt Servo | D10 |
| OLED | I²C |

Table 3.1 – Pin configuration for the V3 demonstration build

The complete V3 design additionally includes the robotic movement (wheel) system, which is not part of the current demonstration.

## 3.3 Software Architecture

The software is broken down into functional sections to keep the codebase manageable and easier to modify:

- Distance measurement

- Object detection

- Servo control

- Smooth pan movement

- Smooth tilt movement

- Facial expressions

- Blinking

- Looking left

- Looking right

- Idle behaviour

Splitting the program this way made it far easier to test one behaviour at a time and to trace bugs back to a specific function rather than the sketch as a whole.

# Chapter 04 – Implementation

## 4.1 Arduino Programming

NYX's firmware is written in Arduino C/C++, using four main libraries:

- Wire – for I²C communication

- Adafruit GFX – for graphics primitives

- Adafruit SSD1306 – for driving the OLED

- Servo – for controlling the pan and tilt motors

## 4.2 Ultrasonic Distance Measurement

The ultrasonic sensor is wired to pins D2 (trigger) and D3 (echo). The Arduino sends a 10-microsecond trigger pulse and then measures the returning echo with pulseIn(). That duration is converted into centimetres using:

distance = duration × 0.0343 / 2

A timeout is also built in so the program does not hang if no echo is received – something I ran into early on when testing with objects placed too far away or at awkward angles.

## 4.3 Object Detection

NYX uses a detection threshold of 40 cm. When the measured distance is 40 cm or less, the system treats that as an object being detected, and the robot:

- Displays the curious expression

- Moves its head

- Looks left

- Looks right

- Returns to the centre position

When the measured distance is greater than 40 cm, NYX simply stays in its normal, idle state.

## 4.4 Servo Movement

The pan servo handles horizontal movement and the tilt servo handles vertical movement. Instead of jumping straight to a target angle, NYX steps the servo angle gradually, which produces much more natural-looking head motion – the difference between the two approaches was obvious the first time I tested it, the smooth version genuinely looked more “alive”.

## 4.5 OLED Facial System

The OLED is what gives NYX most of its personality. The expressions currently implemented are:

- Normal

- Blink

- Look Left

- Look Right

- Happy

- Super Happy

- Curious

- Worried

Each of these is triggered by a specific state in the behaviour logic, so the face always reflects what the robot is currently doing.

## 4.6 Body Movement

The complete NYX V3 design includes body movement driven by motor control hardware, with the following intended behaviours:

- Forward

- Backward

- Left turn

- Right turn

- Stop

This part of the system is currently disabled in the demonstration version because the movement power hardware was damaged during development. The code and design for it remain part of the complete V3 architecture, and I intend to bring it back once the damaged components are replaced.

# Chapter 05 – Testing and Results

## 5.1 Testing Methodology

I tested NYX incrementally rather than waiting until everything was assembled. Each component – the ultrasonic sensor, the OLED, the servos, the facial expressions, head movement, body movement and the IR sensors – was checked on its own first, before being integrated with the rest of the system.

## 5.2 Ultrasonic Testing

I tested the HC-SR04 by placing objects at different distances from the sensor and watching the readings on the Serial Monitor. A typical clear-path reading looked like this:

Distance: 80.5 cm

Clear path

And when an object was brought closer:

Distance: 25.2 cm

OBJECT DETECTED

This confirmed that the sensor was providing genuine, real-time input rather than the behaviour being driven by simulated or hard-coded values.

## 5.3 Servo Testing

The pan and tilt servos were tested individually before being tied into the detection logic. Both moved reliably between their programmed positions and delivered the head movement the interactive behaviour depends on.

## 5.4 OLED Testing

I tested the OLED against every expression NYX currently supports, and the display switched correctly each time the robot's state changed.

## 5.5 Integrated Testing

With the ultrasonic sensor, OLED and head servos all connected together, the integrated behaviour worked as designed:

| Condition | Resulting Behaviour |
| --- | --- |
| Clear environment (distance > 40 cm) | Normal / idle expression, head stays centred |
| Object within 40 cm | Curious expression, head turns toward the object |

Table 5.1 – Integrated test results

This confirms the core interactive functionality that Project NYX set out to demonstrate.

## 5.6 Testing Limitations

The one thing I could not demonstrate was physical body movement, since the movement power system was damaged before this stage of testing. As a result, the wheel motors are not activated in the presentation version. This does not affect the ultrasonic sensing, OLED display or head servos, which all continue to work exactly as intended.

# Chapter 06 – Conclusion and Future Development

## 6.1 Conclusion

Project NYX shows what an interactive robotic system built around an Arduino microcontroller can look like when sensing, movement and visual feedback are designed to work together rather than sitting as separate demos. The final version I am presenting brings together a real ultrasonic sensor, an OLED display and two servo motors: NYX can measure the distance to an object, decide whether it falls inside the detection range, and respond with a facial expression and head movement that make that response legible to a person watching.

Just as importantly, the project reflects an incremental development process, moving from basic sensor-and-servo control in V0 through to the more complete interactive behaviour in V3. Although the body movement system could not be demonstrated because of hardware damage, the complete V3 architecture already includes that functionality, and it can be restored once the damaged components are replaced.

## 6.2 Achievements

- Built a working Arduino-based robotic controller

- Implemented real ultrasonic distance measurement

- Implemented 40 cm object detection logic

- Implemented pan servo control

- Implemented tilt servo control

- Achieved smooth, natural-looking head movement

- Built an OLED-based facial interface

- Implemented multiple facial expressions

- Achieved an interactive object-response behaviour

- Implemented autonomous idle behaviour

- Structured the software in a modular way

- Carried out component-level testing before integration

- Delivered a working, integrated robotic prototype

## 6.3 Limitations

- Body movement cannot currently be demonstrated.

- The original movement power system was damaged.

- Advanced object recognition is not available.

- The ultrasonic sensor only provides distance information, nothing about what the object actually is.

- Behaviour is based on predefined programming rather than any form of AI or learning.

## 6.4 Future Improvements

If I continue developing NYX beyond this report, the areas I would prioritise are:

- Replacing the damaged movement power system

- Restoring full autonomous body movement

- Adding multiple ultrasonic sensors for wider coverage

- Re-integrating IR edge detection

- Adding obstacle avoidance

- Adding Bluetooth or Wi-Fi communication

- Adding voice recognition

- Adding a speaker for sound effects

- Adding camera-based object recognition

- Exploring simple artificial intelligence for behaviour

- Adding autonomous navigation

- Building more advanced OLED animations

- Improving power management with a rechargeable, protected supply

## 6.5 Final Conclusion

NYX demonstrates, on a small scale, how sensors, a microcontroller, actuators and a graphical display can be combined into one interactive robotic system. The current V3 demonstration proves out the core interaction using real ultrasonic distance sensing, real servo movement and the existing facial-expression system, and the complete movement architecture gives me a clear foundation to build on once the damaged hardware is replaced.

# References

1. Arduino Documentation.

2. Adafruit GFX Library Documentation.

3. Adafruit SSD1306 Library Documentation.

4. Arduino Servo Library Documentation.

5. HC-SR04 Ultrasonic Sensor Technical Documentation.

6. Project NYX GitHub Repository.

7. NYX V0, V1, V2 and V3 development source code.

# Appendix A – Current V3 Demo Pin Configuration

| Component | Pin |
| --- | --- |
| HC-SR04 TRIG | D2 |
| HC-SR04 ECHO | D3 |
| Pan Servo | D9 |
| Tilt Servo | D10 |
| OLED SDA | Arduino SDA |
| OLED SCL | Arduino SCL |

Table A.1 – Full pin configuration for the demonstration build

# Appendix B – V3 Operating Logic

The operating logic follows a simple, repeating sequence once the system starts up:

| Step | Action |
| --- | --- |
| 1 | Initialize Arduino |
| 2 | Initialise OLED |
| 3 | Initialise servos |
| 4 | Read HC-SR04 sensor |
| 5 | Calculate distance |
| 6 | Check whether distance ≤ 40 cm |

Table B.1 – Startup and measurement sequence

Depending on the result of that check, the robot follows one of two branches before the whole sequence repeats:

| Condition | Expression | Movement |
| --- | --- | --- |
| Distance ≤ 40 cm (object detected) | Curious expression | Head movement toward the object |
| Distance > 40 cm (clear path) | Normal / idle expression | Head movement resumes idle pattern |

Table B.2 – Decision branches (sequence repeats continuously)

Current presentation configuration: real ultrasonic sensing, real OLED display and real pan/tilt movement, with body/wheel movement disabled because the movement hardware is currently unavailable.